[image: image1.png]Door Controller Settings

Edit Box AP name

ooy AP password

Edit Box Business name

Edit Box User name

Edit Box Login password

Edit Box Door strike activation(seconds)
Edit Box Tail strike activation(seconds)

save




	Copyright 2017 Coral Springs Software.  All rights reserved.


[image: image2.jpg]Connect the TAIL GATE system here

Connect the ELECTRIC DOOR STRIKE here

Connect the EMERGENCY DOOR BUTTON here

[J8]0C12V | A
37 TAIL I 405A GND | | TAIL BTN
[36|000R {| c ‘ ‘

74| R488|

| ——€ARD READER
Onnamenn,
-~ JS

Connect the POWER SUPPLY here Connect the SERIAL BARCODE SCANNER here





DC-Q1000 Control Access
Manual 

Introduction:

The DC-Q1000 is a custom-designed relay controller developed by Coral Springs Software.  The purpose of this device is to provide a suitable door interface access/tailgate system in conjunction with the Coral Springs Software QuickChek cloud application.  
Package Content

· 1 – DC-Q1000 Access Controller

· 1 – 12 VDC Power Supply

· 1 – Serial Barcode Reader with 5 VDC power supply
· 1 – Mounting Plate
Compatibility:

The DC-Q1000 was developed for compatibility with the Coral Springs Software QuickChek application.  

Hardware Installation:

This product is intended for professional installation by a qualified electrician or a low voltage specialist.
The DC-Q1000 is easily installed.  Open the enclosure by removing the retaining screws and detaching the lid.  Locate the field-wiring terminal connector.  Attach door, tailgate, and optional door-exit wires as required.  Apply +12VDC power to the controller by plugging in the adapter at the barrel plug.  Attach one end of the host interface cable (DB9) to the Serial connector then connect to Serial barcode scanner. Appropriately route external wires and cables. Re-attach enclosure lid and secure with retaining screws, being careful not to pinch or bind cable(s).  

The unit can be secured to a wall or other surface.  (Mounting screws and/or other special hardware may be required - included.)  For installation of the 12VDC electronic door strike and hardware please contact your local locksmith.
TB-1 Terminal Connections: 
The DC-Q1000 contains a field-wiring block with the following screw terminal connections:
1) Ground 

2) +12V Out 

3) Ground 

4) Tailgate contact (N/O)

5) Tailgate contact (N/C)

6) Ground  

7) Door Relay contact (N/O)  

8) Door Relay contact (N/C)  

9) 485B Other(N/C)

10) 485A Other(N/O
11) Ground

12) Tailgate override
13) Button

14) Emergency Door override
15) Button

16) Ground
17) +5V Out

Note:  Only a subset of the above connections may be required in your installation  

POWER SUPPLY: 
PLEASE BE SURE TO USE THE CORRECT POWER SUPPLY FOR EACH DEVICE
5V is used for the BARCODE SCANNER

12V is used for the DOOR CONTROLLER BOX 

Setting up WiFi

1. Set SW1 to 1-On 2-Off for operation. 

2. Set SW2 to 1-On 2-Off for operation. The settings for SW2 should never be modified.
3. Set SW1 to 1-Off 2-Off to setup WiFi settings. 2 yellow lights will be on showing that you are in Wifi Setup

4. Enter your local Wifi settings
Accessing Firmware Screen

1. Set SW1 switches to turn on Wifi configuration 1-Off 2-Off
2. Plug in power supply…two yellow lights will be on
3. With your mobile phone or tablet, select Wifi  "Coral Springs AP".
4. Enter Wifi Password 12345678
5. Must use Chrome on your cell phone and go to 192.168.4.1 that will load the firmware settings screen
6. Enter your local WiFi name
7. Enter your WiFi password

8. Enter your QCCloud Business name for login

9. Enter QCCloud Username

10. Enter QCCloud Password then Save

11. Set number of seconds you want the door to stay open in Door Strike Activation

12. DISCONNECT POWER FROM THE DEVICE

13. Return SW1 to operational settings 1-On 2-Off

14. Reconnect Power supply and the green light should stay solid/not blink if it’s connected to your local WiFi

15. Scan a barcode and the device should beep and trigger door lock

16. If member is not allowed in, the device will return a buzzing noise


Care and Maintenance:

The product requires no special care or maintenance other than protection from direct exposure to hazardous or dangerous environments.  It is also vital to shield the controller from exposure to the weather.   

Warranty:

This product is warranted for a period of 1 year from the date of purchase and is guaranteed to be free from defects.  The warranty covers the entire unit, except if any part or component has been modified or otherwise converted from its original form for improper installation.  The warranty does not cover damage or failure due to neglect or improper use.

Disclaimer:

Coral Springs shall not be held liable or responsible for incurred damage or injury which may result due to improper use of this product.

Contact Information
Should you have any questions or comments please contact us at: 954-575-7160
Technical Support Contact:  954-575-7160
Command 12VDC Door Strike Installation with Tailgate System.
Note: Use NO and GND for Electric strikes. Use NC and GND for Magnetic Locks.






How the Wireless Door Controller Works:
1. Member scans
2. Barcode sent to door controller
3. Door controller contacts web server to verify if member is allowed in or not
4. Door controller triggers door if member is active and has a zero balance

Coral Springs�
DC-Q1000 Access Control Manual�
�






Serial connector





Crash Bar for free exit





Serial Cable from Barcode Reader      DO NOT CUT


(See diagram below)











Inside looking out





Door must swing outward for Tailgate installations.





Outside looking in





Bar Code Card Swipe





Tailgate System





Tailgate sold separately.    


Call Rio Systems for details 763-241-1085








